| Test | Inputs | Outputs | Expected Result |
| --- | --- | --- | --- |
| Scan finds device | Mock contains one valid BLE device | List of discovered devices | Device is returned with correct ID and name |
| Connection succeeds | Valid device ID | Successful connection | Adapter enters connected state and isConnected() returns true. |
| Connection fails | Valid or invalid device ID with mock configured to fail | Connection error | Application remains disconnected and can retry. |
| Unexpected disconnect | Connected device; mock drops connection | Disconnect event/state change | Application detects connection loss and does not remain falsely connected |
| Reconnect after disconnect | Previously connected device becomes available again | Successful reconnection | Communication resumes |
| Read normal device status | Valid DeviceStatus values | Device status object | Protocol version, battery, storage, and pending event count match the device values |
| Unsupported protocol version | Protocol version not supported by app | Device status | Application identifies the incompatibility rather than continuing as though compatible. |
| Send valid settings | Valid images per event | Successful settings write | Mock receives exactly the value requested by the application. |
| Request pending events | Device contains several pending event IDs | Array of event IDs | All valid pending event IDs are returned correctly. |
| Recover missed event | Event occurred while app was disconnected | Pending event ID & event information | Application retrieves the event even though the original notification was missed. |
| Receive normal event notification | Valid EventNotification | Event callback | Correct event ID, distance, time offset, image count, and image format are received. |
| Duplicate event notification | Same event notification sent twice | Two callbacks for same event ID | Application recognizes that both notifications represent the same logical event. |
| Receive valid ImageInfo | Valid image size, total chunks, image index, and format | ImageInfo callback | Application records correct metadata for the correct event and image |
| Single-image transfer | One image split into valid sequential chunks | Completed image data | Chunks are combined into one image in the correct order |
| Multiple-image transfer | Event containing multiple images | Multiple completed images | Each image is reconstructed separately and associated with the correct image index. |
| Missing image chunk | Chunks 0,1,3,4 when chunk 2 is required | Incomplete transfer | Image is not marked complete and missing chunk is detected. |
| Duplicate image chunk | Chunks 0,1,1,2,3 | Transfer data | Duplicate chunk does not cause its payload to be added twice. |
| Out-of-order image chunks | Chunks arrive 0,2,1,3 | Transfer data | Application either reorders them correctly or detects the invalid sequence according to policy. |
| Incorrect last-chunk flag | A middle chunk has isLastChunk = true | Image chunk | Application does not incorrectly treat the image as complete. |
| Incorrect image byte count | All chunks arrive but combined size differs from imageSizeBytes | Reconstructed payload | Image is treated as corrupt/incomplete and is not accepted. |
| Chunk belongs to wrong event | Transfer Event 100. Receive chunk with Event 200 | Image chunk callback | Chunk is never attached to Event 100. |
| Interleaved image chunks | Chunks from Image 0 and Image 1 arrive mixed together | Multiple chunk callbacks | Each chunk is associated with the correct image and both images reconstruct correctly. |
| Disconnect during image transfer | Several chunks arrive, then BLE disconnects | Partial image + disconnect | Partial progress is preserved and image is not incorrectly marked complete. |
| Resume image transfer | Existing partial image. Request starts at first missing image/chunk | Remaining image chunks | Transfer resumes at the correct location without losing or duplicating previously received data |
| Acknowledge completed event | Entire event and all expected images successfully received | Event-transfer acknowledgment | Correct event ID is acknowledged only after the event is fully validated. Incomplete/corrupt events are not processed |